|  |  | Configuration of Permanent Magnet Electric Motor |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 9kW Brushless DC Motor |  |  |  |  |
| NO. |  | Parmeters |  | Units | Amount | Remark |
| 1 |  | Rated Power |  | kW | 9 |  |
| 2 |  | Peak Power |  | kW | 10 | Limiting the specific values of the secision by the controller |
| 3 |  | Rated Voltage |  | VDC | 96 |  |
| 4 |  | Rated Current |  | A | 104 |  |
| 5 |  | Rated Torque |  | Nm | 30.7 |  |
| 6 |  | Overload Torque |  | Nm | 61.4 | Limiting the specific values of the secision by the controller |
| 7 |  | Rated Speed |  | rpm | 2800 |  |
| 8 |  | Max. Speed |  | rpm | 3400 |  |
| 9 |  | Anti-Electromotive |  | V/rad·s-1 | 0.24 |  |
| 11 |  | Working Type |  |  | 1h |  |
| 12 |  | Protection Grade |  |  | IP55 |  |
| 13 |  | Insulation Level |  |  | F |  |
| 14 |  | Cooling Mode |  |  | NATURE |  |
| 15 |  | Frame |  |  | H132-B35 |  |
| 16 |  | Weight |  | kg | 32 |  |
| 17 |  | Appearance |  |  |  |  |
|  |  |  |  |  |  | Practical requirements of motor output shaft can be customized |
| 18 |  | Size | mm |  | Φ248×307 |  |
| CONTROLLER SPECS. |  |  | BSC20-72 |  |  |  |
| INPUT VOLTAGE RANGE 24-120 COOLONG TYPE NATURE |  |  |  |  |  |  |
| MAX OUTPUT CURRENT 400A WORKING TEMPERATURE -20-60 |  |  |  |  |  |  |
| CONTINUES CURRENT 200A |  |  |  |  |  |  |
| APPEARANCE AS FOLLOWING : |  |  |  |  |  |  |
| KEEP TEMPERATURE -30-80 WEIGHT 4 KG |  |  |  |  |  |  |

| PEDAL |  |
| --- | --- |
| INPUT VOLTAGE | 0.8--4.2V |
| CONTROL TYPE | POTENTIONMETER |